1.	General Requirements

An automated follow system based on UWB (Ultra-wideband) radio tracking shall be supplied.

The automated follow system shall be provided with multiple POE powered antennas, trackers with a battery lifetime greater than 3,5 hours and a main server, with a capability of a backup server, using a standardized Ethernet network for inter-communication. Trained operators shall be able to swap single devices in the case of a fault. Under no conditions shall a single faulty device affect the remaining system or parts of it.

2.	Automated Follow System Implementation

A versatile automated follow system shall be provided, suitable for the operating requirements of theatres, television broadcasts, special events, stadium shows and live performances. The main focus of the automated follow system shall be the control of moving lights, audio servers, video servers and conventional luminaires (also called fixtures).

3.	Basic System Design

The system design and architecture shall be based on the following devices:

- server with an integral logic board, an embedded operating system

- optional backup with an integral logic board, an embedded operating system

- UWB (Ultra-wideband) antennas distribution around the tracking are via Power over Ethernet using a closed network

- UWB (Ultra-wideband) trackers placed on moving targets to calculate their real-time positions

- DMX distribution via Ethernet using Ethernet DMX nodes or DMX over Ethernet protocols

4.	Hardware Design

SERVER

General

A single server shall be able to control Lighting-, Audio- and Video Fixtures. The Control of these Fixtures should be done over standardized Ethernet protocols. A second server should be able to work as a complete backup of the system.

Dimensions and Weight

The server shall have a lightweight chassis designed as a 19” server rack with 1 Unit.

The weight shall not exceed 5 kg / 11 lbs.

The maximum dimensions shall be

480 x 249 x 44 mm / 19 x 9,8 x 1,7 in

(width x depth x height)

Central Processing Unit and Storage

The server shall incorporate a manufacturer-designed processing engine featuring high performance AMD EPYC technology. The minimum requirements for the main CPU are a Quad-Core with 64 Bit and a minimum of 16 GB RAM.

The server shall have an integrated SSD drive with a minimum capacity of 1 TB to store the operating system and show files.

Connectors

The server shall support a variety of peripheral equipment. Therefore, the ports listed below shall be integral by default:

1 x IEC Socket

5 x RJ45 Connectors

2 x USB 3.0 (type A)

1 x VGA Connector

UWB ANTENNAS

General

The UWB antenna shall be an active tracking device. For user feedback there shall be a colored LED in the front, which interact with the software.

Dimensions and Weight

The anchor shall have a lightweight water-resistant chassis designed for mounting on a rig.

The weight shall not exceed 0,5 kg / 4,4 lbs.

The maximum dimensions shall be

190 x 150 x 38 mm / 8,6 x 8,8 x 1,7 in

(width x depth x height)

Power Supply

The power supply shall be designed for operation with POE 802.3 af, with a maximum power of 15W.

Connectors

The anchor shall support a robust ethernet connection.

TRACKER

General

The tracker shall be an active tracking device. For user feedback there shall be a colored LED, which interacts with the software. The tracker shall not be able to be switched off by the user.

Dimensions and Weight

The tracker shall have a very lightweight case with water-resistancy without any openings.

The weight shall not exceed 16 g / 0,035 lbs.

The maximum dimensions shall be

47 x 33 x 7,5 mm / 8,6 x 8,8 x 1,7 in

(width x depth x height)

Power Supply

The tracker shall be equipped with a rechargeable battery with a capacity of at least 3,5 hours in the tracking area. The battery shall not be exchangeable to ensure water resistance.

TRACKER CHARGING STATION

Dimensions and Weight

The charging station shall provide at least 30 charging slots. It should be able to charge the tracker in less than 2 hours.

The weight shall not exceed 2 kg / 4,4 lbs.

The maximum dimensions shall be

295 x 235 x 20 mm / 11,6 x 9,25 x 0,79 in

(width x depth x height)

Power Supply

The power supply shall be designed for operation with a DC Socket, with a maximum Power of 60W.

Connectors

The Charging Station shall use a DC Connector for Power Supply.

5.	Operating and Software

General Operation

The system shall be able to track the trackers in full 3D space. The capable tracking area should be 50m x 50m x 20m. The server should handle up to 20 anchors to ensure best tracking results and enough redundancy. The User shall be able to set up the system easily with a guided wizard.

The usage of the system shall be done with the Client App Version working on an Android Tablet. To ensure proper functions the client shall work in a Client Server Architecture with dedicated show-file up -and downloads.

Software updates shall be able to be done via a web Interface via a network connected PC or Tablet. The update packages shall be provided free of charge from the manufacturer.

A real-time version of Linux shall be used as the operating system.

A single server shall provide a tracking information to control fixtures (light, audio, video) simultaneously. With a connected second server these should be a backup. The system shall be able to handle different other protocols (sACN, ArtNet, OSC, PSN)

The system shall have the capability to use up to 50 active trackers. The system shall have at least a refresh rate of 15Hz.

7.	Manual and Documentation

The manufacturer, supplier or the vendor must provide an installation manual and an operation manual in English.

8.	Service, Maintenance and Repair

At time of order placement, bidders shall nominate a company and/or a person for service, maintenance and repair for the offered tracking system. This company and/or person must be familiar with the construction and design of the offered tracking system and must work on behalf of the manufacturer.

The systems manufacturer shall grant a 2-year warranty to the device.